9$(h1ii$)	$(kurz9Cubicle system D-Bambino for adults

$(langMade by KEMMLIT-Bauelemente GmbH
Maltschachstr. 37
72144 Dusslingen
Telephone: +49 (0)7072 / 131-0
Telefax: +49 (0)7072 / 131-150
info@kemmlit.de - www.kemmlit.de


Construction:
Toilet cubicles for nursery and infant schools.

Accident safety guaranteed according to

the safety guidelines of the GUV. TÜV-approved.

(GS seal of approval). 

Height:
1970 mm incl. 120 mm under panel clearance.

Also available with 1400 mm door height including 120 mm

under panel clearance.

Partitions and pilasters (front panels):

30 mm thick solid melamine-coated particle board.
All visible edges are protected against impact with

ABS lipping 3 mm thick (radius = 2 mm).

The bottom corners in the doorway are rounded

with a radius of 33 mm to reduce risk of injury.

# Profiles:

Aluminium profiles are rounded with a radius of 2 mm

to prevent injury. The stabilisator/head rail with the dimensions

55/43mm runs above the door fronts and is elegantly rounded.

Wall connection with U-channels to allow minor adjustments for

tolerances on site.

Supporting legs:
Height adjustable aluminium supporting legs 20 mm in diameter,

powder-coated with stainless steel plate and durable nylon cover sleeve.

Doors:

Same construction as partitions and pilasters (front panels).

Doors sit flush with front. The corners are rounded with a

radius of 33 mm to prevent injuries.

Hardware: 3 options available:

- 2 robust 2-roller edge hinges made of aluminium, 
20 mm in diameter. Double-latch lock WC. Safety lever handle
23 mm in diameter. Red/white indicator bolt with an emergency

release mechanism made of impact resistant nylon.

- 3-roller spring loaded edge hinge, 20 mm in diameter,

with aluminium finger protection profile running between the

hinges. Double-latch lock WC. Safety lever handle 23 mm in

diameter. Red/white indicator bolt with an emergency

release mechanism made of impact resistant nylon.

- 3-roller spring loaded edge hinge, 20 mm in diameter,

with aluminium finger protection profile running between the

hinges.	Soft finger protection profile on the lock side.

Safety ring handle made of impact- and crack-resistant nylon

with integrated rubber buffer as door stop. Therefore door opening

outwards only. If required with lever lock, colour: white.

For option 2, a door height of 1400 mm, suitable for

children including 120 mm under panel clearance is also available.

Colours:

Colours of pilasters (front panels), partitions and

doors according to the KEMMLIT colour charts

(at least 15 colours)

Profiles, hardware, door handles, supporting legs and

accessories are available in the same colours according to

the KEMMLIT colour charts (at least 6 colours).

All prices ex works in Dusslingen. Taxes and duties

are unpaid, materials only. Installation and transportation

are not included.

9.10		0,000 no.             .............$)	                           $(gb.............$(ep$)

Accessories:

9.20             	KEMMLIT door stopper hooks made of nylon 301$)

0,000 no.             .............$)	                           $(gb.............$(ep$)

9.30             	KEMMLIT wall hooks made of nylon 302$)

0,000 no.              .............$)	                          $(gb.............

/E$$NNN

$(h1ii			$)	$(menge $)                    KEMMLIT paper roll holder made of nylon 308$)

$(d276$)	9.40                                                                                         0,000 no.              .............$)	                           $(gb.............$(ep$)

$(lang

KEMMLIT replacement paper roll holder made of nylon 309/E

9.50$$NNN

$(h1ii$)	                       $(menge0,000$) no.             $(einh$)	$(d276$)$(ep.............$)	  $(gb.............$)

$(lang$)

/E$$NNN

$(h1ii                    $(menge $)	$(langKEMMLIT toilet brush set, square-shaped, made of nylon 310$)

$(d276$)

9.60		                       0,000 no.             .............$)	                           $(gb.............$(ep$)

/E$$GRUEnd

Accessories also available in stainless steel

----------------------

Cubicle System D-Bambino for adults$)	$(gb.............$)

============

/E